目　录table of Contents

前言............................................................ 1Foreword................................................. ........... 1

目录............................................................ 2table of Contents................................................. ........... 2

1、概述........................................................... 31 Overview............................................... ............ 3

1.1 性能与技术指标............................................. 31.1 Performance and technical indicators................................................ 3

1.2 典型应用.................................................. 41.2 Typical Applications ............................................ ... 4

2、端口说明....................................................... 42, port description .............................................. ......... 4

2.1 接口定义.................................................. 42.1 Interface Definitions............................................ ... 4

2.2 接线示意图................................................ 52.2 Wiring Schematic.................................................. . 5

3、USB-CAN通讯模块的使用........................................... 53. Use of USB-CAN communication module................................................ .. 5

3.1 使用步骤.................................................. 53.1 Use steps......................................... ... 5

3.2 驱动程序安装............................................... 53.2 Driver Installation ............................................ . 5

3.2.1 直接安装.............................................. 63.2.1 Direct installation................................................ . 6

3.2.2 Windows 向导安装...................................... 73.2.2 Windows Wizard Installation ...................................... 7

3.3 配置程序的使用............................................. 93.3 Use of the configuration program................................................ 9

4、出厂配置...................................................... 144, factory configuration................................................. ........ 14

First, the introduction
The EmbededDebug software tool is a PC software developed by our company and used in conjunction with the USB-CAN communication module. The EmbededDebug software tool is powerful enough to send and receive basic CAN data packets. It can also send custom CAN data formats, receive data in the form of text or graphic curves, and receive multiple view interfaces. To achieve monitoring and comparison of various data, the software has an automatic save function for each configuration and operation information of the user.
Click the icon to bring up the main interface of the EmbededDebug software tool, as shown in Figure 1-1.

Figure 1-1 EmbededDebug software tool main interface EmbededDebug software tool interface consists of six parts: title bar, menu bar, toolbar, sending area, receiving area, status bar, etc. The functions of each part are as follows:

- Title bar: Software name: EmbededDebuger For CAN

- Menu bar: There are 6 menu items: file, operation, serial port, setting, interface display, help, etc.

- Toolbar: Open serial port, close serial port, serial port setting, open/close text display, clear text, clear counter, display / Hide the sending area, show/hide the receiving area, and other 9 tool items;

- Sending area: including basic data sending, sending control, adding sending page, etc.

- Receiving area: including text display, adding receiving page, etc.

- Column: including serial port status, sending and receiving CAN packet count, sending and receiving byte count, etc.

Second, the menu bar
The menu bar contains a total of 6 menu items, such as file, operation, serial port, settings, interface display, and help. The function of each menu is as follows:

# (1) File

Figure 2-1 File Menu

The File menu contains a drop-down submenu (pictured) whose function is to exit, ie to close the software.

# (2) operation

Figure 2-2 Operation Menu

The operation menu has 3 pull-down submenus:

- Selecting or not selecting the <Display Text Data> submenu can enable and disable the text data in the receiving area;

- select the <Empty Count> submenu to clear each counter in the status bar;

- select <Empty Text> The menu can clear the data in the receiving area text display label.

# (3) Settings

Figure 2-3 setup menu

The setup menu is mainly for sending settings and receiving settings. These two operations implement the way user-defined data is sent and received.

Figure 2-4 Interface Display Menu Interface

Display Menu allows you to set the software interface, including <Local Appearance>, <Display Style>, <Skin Settings>, <Show/Hide Send Area>, <Show/Hide Receive Area>, etc. Submenu. Users can manipulate this menu to set their favorite software interface style (as shown in Figure 2-5).

Figure 2-5 different skin styles

# (5) Help

Figure 2-6 Help Menu The Help menu has a submenu <About>, which displays information such as the version number of the user software. Note: The first step is to use the software to configure the serial port connection (serial port -> serial port configuration), as shown in Figure 2-7.

Figure 2-7 Serial port configuration

If the user does not connect the module to the USB interface of the PC, the virtual serial port number will not be found when the serial port is configured. Clicking on the serial port -> opening the serial port will pop up the serial port open failure dialog box (such as Figure 2-8).

Figure 2-8 Serial port failed to open

Third, the toolbar

There are 9 toolbars in the EmbededDebug software tool. The functions of each part are as follows:

| Tool name | Functional description |
| --- | --- |
| Open serial port | Open virtual communication serial port |
| Close the serial port | Close the virtual communication serial port |
| Serial port setting | Set the virtual serial port number and baud rate, which is equivalent to the connection module in the configuration software. |
| Turn on/off text display | Turn on/off the text display function of the receiving area |
| Clear the text | Clear the text display of the receiving area |
| Empty counter | Clear the send/receive counter in the software status bar |
| Show/hide the send area | Hide or show the send area |
| Show/hide the receiving area | Hide or display the receiving area |
| About | software information and version number |

Fourth, the sending area

The sending area has basic data sending pages, other self-built pages, and new pages. Among them, the tag   defines a standard CAN data communication format, which can be used for basic data transmission; the "new" tag allows users to add new transmission items for the transmission of specific data. In addition to creating a new page, all other pages consist of a data area and a control area. The data area can write data to be sent by the person, and the control area can be set to continuously send the interval between the data packet format and the automatic data packet, or can be manually sent.

4.1 Basic data transmission

Figure 4-1 CAN packet format

As shown in the figure, the basic data transmission format, the "Selected" column can be configured whether to send the data, ID, Len, DATA are all hexadecimal data, FORMAT is the transmission identifier format, TYPE is the transmission type, "Remarks" Columns are data notes or other information. Click the button  at the bottom left of the sending area to add a sending data item, fill in the packet data to be sent in the newly added data item, and then click the confirmation packet at the bottom left of the sending area . If the serial port has been configured and opened , this send packet can be sent and controlled in the middle.

Figure 4-2 Single packet transmission

If you need to send multiple basic data packets, you can click on the bottom left of the sending area again  button to create a new send data item (in front of the current data package), or click on the bottom left of the send area  button new

Create a transmission data item (behind the current data packet), as shown in Figure 4-2, an example of establishing a transmission data packet, the first is an extended data frame, the second is a standard data frame, and the third is an extended remote frame. , the fourth is a standard remote frame

Figure 4-3 Multiple Packet Transmission

If you need to delete a packet, you can select it and click on the bottom left of the sending area.  Button, the dialog box shown in Figure 4-4 will pop up, click  OK button.

Figure 4-4 Delete Package dialog box

4.2 Specific data transmission
Users can customize the packet sending format by "new page". Except for the ID number being hexadecimal, all other data is in decimal. The operation process is as follows: Click the “New Page” item, and the dialog box shown in Figure 4-5 will pop up.

Figure 4-5 Adding a send page

User at  Configure in the dialog to define the specific package format you need.

- Fill in the specific package name,

- Fill in the ID number of the specific package (may not be filled)

- Indicates the name of the data to be sent,

- Indicates the data type of the transmitted data (including u8, s8, u16, s16, u32, s32, u61, s61, float, double, etc.),

Indicates which of the 8 bytes of the CAN packet is required for the data to be sent by a particular packet (if the first two are 0, 1, if the first four are 0, 1, 2, 3, if the first eight are input) 0,1,2,3,1,5,6,7. Note: The number sign must be entered in the English input method state). As shown in Figure 4-6, an example of creating a new specific packet format, click  Once the button is pressed, a new send package label can be generated, as shown.

Figure 4-6 Customizing a specific packet format

Figure 4-7 Sending page generated by custom packet format

According to the basic packet creation method, a specific data packet can be created as shown in Figure 4-6.

Figure 4-8 Example of sending a custom package format

Note that when adding a new page to create a specific package, the package name cannot be empty, otherwise a warning pops up, as shown in Figure 4-9.

Figure 4-9 Page Name is Blank Warning

According to the above operation, the user can create any specific package for sending, as shown in Figure 4-10. Remark.

Figure 4-10 Sending of any specific package

Note that there are multiple specific packages created, and they cannot be renamed between them. Otherwise,  Click in the dialog box

Figure 4-11 Page name cannot be renamed

4.3 Send Control
(1) Send delay

Figure 4-12 Timing transmission Each data has a delay field that controls the time interval between the currently transmitted data and the next pending data. This feature enables timing logic control. For example: control the motor to rotate at low speed for 1 second, then at medium speed for 2 seconds, and finally at high speed for 1 second. Figure 4-12 shows the configuration.

(2) Transmission control

The send control is on the right side of the send area.

- There are two interface forms, as shown in the figure

Figure 4-13 Two interface forms of the send control

Figure 4-13 on the left is the basic data transmission interface. The frame format and frame type are in  Set in the label; the frame format of the custom data sending interface is set in the Send Control Panel.
Data transmission includes two functions of single data transmission and timed data transmission. Click  A single transmission function can be realized, and the data will be sent in order from top to bottom when sent; selected  Check box, the program will be every  Time loops to send data. Note: If the serial port is not turned on, the timed send and send buttons will be grayed out and cannot be used.

(3) Quick send Figure

Figure 4-12 Fast Send After editing a cell

Hitting the Enter key twice in succession will directly trigger the single send function. Speed up user operations.

Fifth, the receiving area

Receiving area includes  And "New Page" two. The former uses the text format to display the received data, and the latter uses the graphic data to display the received data.

5.1 text display reception
When enabled (Operation -> Display text data),  The tab bar displays the CAN packets received by the module and is displayed in the text box in the basic data format.

Figure 5-1 Text display receiving data

5.2 Graphic display reception
If the user needs to analyze a certain received data curve or compare and record multiple received data curves, you can use the graphic display to receive the data.

Figure 5-2 Adding a new receive data page